Putting Together a Typical Program

A typical data acquisition program has three parts:

1. Initialization

2. Set-Up

3. Measurement

***    Use the cursor keys to browse through this document.   ***

***    Press Esc to return to the MENU.                       ***

For this demonstration session, BASIC commands have been used to send

the command strings to the 488-PC1A.  The BASIC commands can be replaced

by input-output commands from many other programming languages without

affecting the 488-PC1A's operation.

INITIALIZATION is done to establish communications between the PC

and the 488-PC1A and to reset all bus devices and ensure that

they are in a known state.  The following is a typical initialization

sequence:

10 REM                   'load call routine addresses

20 DEF SEG=&HC000

30 INIT%=0    :OUTPUT%=3  :ENTER%=6   :ABORT%=9    :EOL%=12 :    DEVCLR%=15

40 LLO%=18    :LOCAL%=21  :PPOLL%=24  :PPOLLC%=27  :PPOLLU%=30  :REMOTE%=33

50 SEND%=36   :SPOLL%=39  :STATUS%=42 :TIMEOUT%=45 :TRIGGER%=48

60 ENTERA%=51 :OUTPUTA%=54:DEVICE%=57 :ERRPTR%=60

70 REM

80 REM                    '  reset interface

100 CALL ABORT%

110 CLS                   '  clear crt

SET-UP is the process of adjusting the instrument to take the correct

reading or setting the stimuli to output the correct signal.  It may

also be switching relays to configure the test or turning on the

correct power supplies.

In our example, we are just going to output set-up commands to a

multimeter so it can measure a voltage.  The set-up commands vary for

each instrument and are listed in the instrument's manual.  For our

sample multimeter, the commands are:

F1 = VDC

R1 = 30 volt range

N5T4Z1D1 = balance of set-up for triggered operation.

When put all together, the multimeter command setting looks like this:

200  ADDR%=2:DATA$="F1R1N5T4Z1D1"    ' Writes out the

210  CALL OUTPUT(ADDR%,DATA$)        ' set-up command to device 02.

MEASUREMENT is the process of taking the reading and sending the

data to the computer.  Since the sample multimeter was set to take a

reading each time it was triggered, the program should output the

trigger command once everything has been set-up and stabilized.

300 CMD$="UNL UNT MTA LISTEN 02 GET"

310 CALL SEND%(CMD$)                  'trigger the multimeter

(or 300 CALL TRIGGER%(ADDR%)          'trigger the multimeter

or 300 ADDRM%=-1

310 CALL TRIGGER%(ADDRM%)         'trigger the bus)

320 D$=SPACE$(80)                     '

330 CALL ENTER%(ADDR%,D$)             'get response

340 PRINT "RESPONSE=";D$;

350 END

This completes the lesson.

***    Use the cursor keys to browse through this document.   ***

***    Press Esc to return to the MENU.                       ***