G L O S S A R Y

The following terms are used to describe devices interconnected

together by the IEEE 488 Bus.

***    Use the cursor keys to browse through this document.   ***

***    Press Esc to return to the MENU.                       ***

CONTROLLER    A unit that is capable of issuing commands to other Bus

devices.  A system may have multiple controllers but there

can only be one active controller on the Bus at a time.

SYSTEM        The controller that outputs the remote and interface clear

CONTROLLER

messages (REN and IFC lines).  There can be only one system

controller within a system.

CONTROLLER-   Defines the active Bus controller.  In multiple

IN-CHARGE

controller systems, the system controller is normally

the controller-in-charge at power turn-on and passes

control to other controllers as the need arises.

DEVICE         A unit that is connected to the Bus cables.  In automated

systems, most devices can be addressed as a talker and as a

listener.

TALKER        A device that has been addressed by the controller as a

talker.  There can be only one talker active on the Bus

at a time.

LISTENER      A device that has been addressed by the controller to

listen.  There can be several listeners active on the Bus,

all listening to the same instruction or data message.

SERIAL POLL   A means of reading a single eight bit status byte

from a device.  One of the bits shows if the device

generated a service request (SRQ); the meanings of the

remaining seven bits are defined by the device manufacturer.

SRQ           SRQ is a Bus control line that a device uses to

output a service request signal.  The term is often used

loosely to indicate a Bus interrupt.  If more than one

device on the Bus is capable of generating an SRQ, the

controller program will have to serial poll each of the

devices to determine which are generating service requests.

PARALLEL      A means of reading a single eight bit status byte

POLL

from up to 8 bus devices at one time.  Each of the devices

that respond to the poll outputs a status bit on a line

assigned to them by the parallel poll configure command. The

meaning of the status bit is defined by the device

manufacturer and usually is a very generalized status

indication, for example "Printer Ready" or "Printer Busy".

***    Use the cursor keys to browse through this document.   ***

***    Press Esc to return to the MENU.                       ***