About the 488-PC1A

About the 488-PC1A is a tutorial description of ICS Electronics newest

IEEE 488 Bus controller.  With the 488-PC1A, you can turn any IBM

compatible personal computer into an inexpensive yet powerful GPIB

controller.  The 488-PC1A interface software and working memory reside

on the board in ROM and RAM so you do not use any of your system memory

and do not require disk software!

***    Use the cursor keys to browse through this document.   ***

***    Press Esc to return to the MENU.                       ***

The tutorial is organized as several lessons, each consisting of

several pages.  The lessons are as follows:

1.   The 488-PC1A Description (this lesson)    (TOUR.DOC)

2.   Glossary of Bus Terminology              (GLOSSARY.DOC)

3.   Putting together a typical program       (TYPICAL.DOC)

4.   How to order a  488-PC1A                  (ORDRINFO.DOC)

The ICS 488-PC1A is an intelligent interface card that may be used with

any PC or PC compatible computer to control or communicate with an

IEEE-488 Bus system.

The 488-PC1A easily plugs in to any empty slot in the PC.  The software

is on the card and becomes an extension of your operating system.

There is also RAM memory on the 488-PC1 card so none of your system

memory is required for operation.  The 488-PC1A provides total support

of all IEEE-488 commands.

As a Bus controller, the 488-PC1A accepts high level instructions from

the computer and then hand-shakes the necessary command and data

characters onto the Bus by way of an NEC 7210 GPIB controller chip.

Device data, status bytes, and error messages are sent back to the host

computer via the same interface controller.  The 488-PC1A also acts as

an addressable device that interfaces the PC to the IEEE 488 Bus.

The 488-PC1A contains the logic and intelligence necessary to act

as a stand-alone Bus controller.  This peripheral processor function

relieves the host computer from executing each Bus instruction and

frees it for other data reduction tasks.  The 488-PC1A does this by

executing assembly language commands sent to it as data strings in a

CALL statement.  Since this or a similar command exists in many

computer languages, the 488-PC1A is programable with many programing

languages.

As a Bus controller, the 488-PC1A is capable of addressing any device on

the Bus, sending and receiving data and generating both addressed and

universal commands.  The 488-PC1A can accept Service Requests, conduct

Serial Polls, and Parallel Polls.  In addition, data transfer commands

enable the 488-PC1A to transparently pass large data blocks to or from

any Bus device using DMA.

The 488-PC1A enables the computer to pass control to another Bus

controller or act as a peripheral device to another controller on the

same Bus. The computer may instruct the 488-PC1A to act as a device

where it can be addressed as a talker or listener by other controllers

on the same Bus.

Flexible format commands guarantee compatibility with virtually all

IEEE 488 devices.  The 488-PC1A includes user aids such as error

messages and status commands to aid the user with their program

development.

Advantages of the 488-PC1A

1.  Converts any compatible PC into an IEEE 488 Bus controller or device.

The 488-PC1As firmware allows it to interface to virtually any

compatible PC, thereby giving that computer control of, or an interface

to, the IEEE 488 Bus.

2.  Independent of your computer's operating system or language.

Because the 488-PC1A contains all software interfaces on the board in

callable assembly code, any programming language that can access the

assembly calls is able to talk to the 488-PC1, regardless of the

computer's operating system or language.

3.  Does not require additional driver programs or floppy disks.

Because the 488-PC1A is a true peripheral processor with its own

built-in intelligence and firmware, it does not require that special

driver or command decoding programs be loaded into the computer.  This

saves computer memory and enhances user convenience.  Just plug the

488-PC1A into your computer and go!

4.  Includes WAIT-STATE switches that allow the 488-PC1A to operate in

the new high speed AT computers.

The Wait State switches allow you to run the 488-PC1A in computers

whose interface card slot clocks run at greater than 8 Mhz. Most of

the older computers ran this bus clock at 5 or 8 Mhz despite the

advertized speed of their CPU clock (4.77 to 16 Mhz). The newer high-

speed computers (20 to 35 Mhz!) are running the interface slot clock

at 10 Mhz or faster.

***    Use the cursor keys to browse through this document.   ***

***    Press Esc to return to the MENU.                       ***