README.DOC

**********

P/N  123013  Revision 3.0

Quick Basic 4.0 or later demo programs                 Dated  8/9/89

I.  INTRODUCTION:

The disk contains three demo programs that run under Microsoft Quick

Basic Compiler version 4.0 or above. Each demo program is written in

multiple modules.  The file PC1ALIB.BAS is used by all three programs,

and it is the module that contains all sub programs used by the

three programs. The file PC1ACOM.BAS is an include file that contains

all COMMON type of variables and data structures. The COMMON SHARED

keyword creates variables shared between the module-level code and all

procedures within that module.

Programs: QB1A34.BAS, QB1AINT.BAS, and QB1ADEV.BAS

--------------------------------------------------

These demo programs show how a PC/XT/AT with the PC1A card can

be used as a bus controller and as a bus device.  The fist demo

program outputs and reads back a data string.  The second program

shows how the PC is programmed to sense SRQ interrupts and input

data in response to the SRQ line.  The third program shows how

the PC is programmed as a device to respond to another IEEE 488

bus controller.

There are  three more demo programs, QB3478.BAS, QB33DEMO.BAS, and

QB34DEMO.BAS, that use a few bus commands.  Look at them for other

programming examples.

In quick basic 4.0 or above, there can be multiple modules in memory.

There are two versions of libraries, the quick (.QLB) version and the

stand-alone (.LIB) version.  Without the .LIB version libraries,

QUICKBASIC can not create a stand alone executable file containing

the file's routines, and users cannot use the .QLB version libraries

inside the environment. It is crucial that users always create the

.LIB version of the library.

To make a .QLB library, make a copy of the  Microsoft QB.LIB library

and use it as a basis for building a Quick library containing all

the routines you need.  When you create a Quick library using LINK,

the library BQLB45.LIB must always be specified after the third comma

on the LINK command line.  When using LINK to create either a .QLB

or a .LIB library, make sure to link the QB.LIB instead of the QB.QLB.

If you only want to run the program from the DOS command line, it is

not necessary to create a .QLB version library.

CAUTION!!!

Remember when building your .QLB or .LIB library, use the BC command

after the DOS prompt to compile your quick basic programs and get

the required .OBJ file.  Don't use modules compiled from inside the

environment, or it will not work.

Refer to Appendix H in Microsoft's QuickBASIC Programming in BASIC manual for

more details.

II. DISK CONTENTS:

******************* .BAS Quick Basic source files    *************************

QB1A34.BAS       Demo data transfer, serial poll, and parallel poll

QB1AINT.BAS      Input data with an SRQ interrupt that makes the PC branch to

an interrupt service routine written in Quick Basic

QB1ADEV.BAS      Operate the PC1A as a device

PC1ALIB.BAS      A library module that contains all the sub programs used by

the three demo programs

PC1ACOM.BAS      An include file module that contains all the global variables

and data structures shared between the main module and the

library module

****************** .EXE files that run under DOS command line ****************

QB1A34.EXE       Demo data transfer, serial polling, and parallel polling

QB1AINT.EXE      Demo Inputting data with an SRQ interrupt

QB1ADEV.EXE      PC1A operating as a device for data transfer with another

controller.

*****************  .OBJ object files compiled with MASM or BC ******************

QB1A34.OBJ       Compiled with BC command from DOS command line from QB1A34.BAS

QB1AINT.OBJ                    "                 "                   QB1AINT.BAS

QB1ADEV.OBJ                    "                 "                   QB1ADEV.BAS

PC1ALIB.OBJ                    "                 "                   PC1ALIB.BAS

PC1ACOM.OBJ                    "                 "                   PC1ACOM.BAS

INT488.OBJ       Assembled w/MASM command from DOS command line from INT488.ASM

It  contains all the utilities for implementing the interrupt

function, and can be linked with other .lib or .obj files

to build the required library file.

******************  .QLB Quick Library file   ********************************

PC1ALIB.QLB  Obtained by the following command after the DOS prompt:

LINK /Q PC1ALIB.OBJ QB.LIB, PC1ALIB.QLB,, BQLB45.LIB;

INT1A.QLB    obtained by the following command after the DOS prompt:

LINK /Q PC1ALIB.OBJ QB.LIB INT488.OBJ, INT1A.QLB,, BQLB45.LIB;

******************  .LIB stand alone library file ****************************

PC1ALIB.LIB  Obtained by the following DOS command:

LIB PC1ALIB.LIB+PC1ALIB.OBJ+QB.LIB

INT1A.LIB    Obtained by the following DOS command:

LIB INT1A.LIB+PC1ALIB.OBJ+QB.LIB+INT488.OBJ

III. CAUTIONS:

For any of the demo programs, QB1A34.BAS, QB1AINT.BAS and

QB1ADEV.BAS, to work properly, you need to pay attention to the

following:

1.  If you use the demo program source code to develop your own

programs, be sure to include the PC1ACOM.BAS file, and use the

appropriate .obj module, such as INT488.OBJ or PC1ALIB.OBJ, to

build your required library module.

2.  The PC1A memory segment switch  has to be set to hex D000 unless

you make the changes in the program to use a different address.

3.  While building your library file, you need to have the following

files available in the current directory or in the LIB path.

LIB.EXE              QB.LIB

LINK.EXE             BQLB45.LIB

4.  Notes regarding programming for INTERRUPTS:

ICS offers a simple and straighforward way to implement the interrupt

function. Just use the INT488.OBJ file to build your .QLB library file

and a .LIB library file.  Then, the user calls CHGVECTOR(IRQ%) and

RESTOREVECTOR(IRQ%) and passes the desired IRQ channel as the parameter

to redirect the PC's interrupt vector. By calling CHGVECTOR(IRQ%), the

desired vector will point to the quick basic interrupt service routine

- QB45ISR. Then, the user codes QB45ISR in whatever way he needs to get

the required service done.  At the end of the main program, call

RESTOREVECTOR(IRQ%) to restore the PC's interrupt vector for the

desired IRQ channel.

The INT488.OBJ file lets the programmer  implement a real time

hardware interrupt function without involving  any low level

programming work.

Last but not least, the IEEE 488 bus routines are not reentrant.

The user should not plan on using the same bus commands that are

used in the interrupt service routine while waiting for the interrupt

to occur.  In other words, the user may only do an interruptable data

processing task while waiting for a bus interrupt.

VI. DETAILS OF THE DEMO PROGRAMS:

/*-----------------------------------------------------------------------*/

/*                                                                       */

/*                              QB1A34.BAS                               */

/*                                                                       */

/*                                                                       */

/*               DATA LOOP BACK TEST OF A 4834 SERIAL INTERFACE UNIT     */

/*               CONTROLLED BY A PC1A CARD                               */

/*                                                                       */

/* Include file: PC1ACOM.BAS                                             */

/*                                                                       */

/* library file: PC1ALIB.QLB if run from inside the environment          */

/*               PC1ALIB.LIB if run outside the environment              */

/*                                                                       */

/*  Purpose    : This program sends out data string to a 4834 channel,   */

/*               and reads the looped back data from the same channel    */

/*               then displays it.                                       */

/*                                                                       */

/*               This program is written for the ICS Model 4834 Serial   */

/*               Interface which is used to connect devices with RS-232  */

/*               or RS-449 serial interface to the IEEE 488 bus. The T1  */

/*               command puts the 4834 into  local loop back test mode.  */

/*                                                                       */

/*  Languages  : Quick Basic Version 4.0 or later.                       */

/*                                                                       */

/*  Remark     : Please pay attention to the following, or the program   */

/*               will not work properly.                                 */

/*                                                                       */

/*      1.     : In the program, "T1" is sent to the secondary address of*/

/*               the 4834 to select the local loop mode, which is        */

/*               necessary for this test.  Don't change it to "T0".      */

/*                                                                       */

/*      2.     : If you run from inside the environment, you need to type*/

/*               QB /L PC1ALIB.QLB, then load QB1A34.BAS, and press      */

/*               ALT R and select Start.                                 */

/*                                                                       */

/*               If run from the DOS command line, just type QB1A34.     */

/*                                                                       */

/*      3.     : library file used:                                      */

/*               PC1ALIB.QLB   If run from inside the environment        */

/*               PC1ALIB.LIB   If run from the DOS command line.         */

/*                                                                       */

/*      4.     : If you change the path "C:\QB45\PC1ACOM.BAS" for the    */

/*               include file, you have to change the include line in    */

/*               all the modules, and recompile and link.                */

/*-----------------------------------------------------------------------*/

/*                                                                       */

/*                              QB1AINT.C                                */

/*                                                                       */

/* Include file:  PC1ACOM.BAS                                            */

/*                                                                       */

/* Library file:  INT1A.QLB if run from inside the environment           */

/*                INT1A.LIB if run from the DOS command line             */                                                                       */

/*                                                                       */

/* This program demonstrates how a 4834 uses SRQ to interrupt PC/AT. It  */

/* uses interrupt vector 0x0f for IRQ7. The vector is redirected to point*/

/* to the new interrupt service routine - QB45ISR.                       */

/*                                                                       */

/*                                                                       */

/*      1.     : The INT488.OBJ module is integrated into the INT1A.QLB  */

/*               and INT1A.LIB files. The user can call CHGVECTOR        */

/*               and RESTOREVECTOR directly.  The only thing to remember */

/*               is that the statements  DECLARE SUB CHGVECTOR(IRQ%)     */

/*               and DECLARE SUB RESTOREVECTOR(IRQ%) need to be put in   */

/*               the module that calls those procedures.                 */

/*                                                                       */

/*      2.     : Users can rewrite the QB45ISR sub program to fit their  */

/*               own needs.                                              */

/*                                                                       */

/*      3.     : Users can use INT488.OBJ file to build their customed   */

/*               .QLB library and .LIB library.                          */

/*                                                                       */

/*      4.     : If you run from inside the environment, you need to type*/

/*               QB /L INT1A.QLB, then load QB1AINT.BAS, and press       */

/*               ALT R and select Start.                                 */

/*                                                                       */

/*               If run from DOS command line, just type QB1AINT.        */

/*                                                                       */

/*      5.     : The PC1A address switch settin is assumed to be set to  */

/*               address 0xD000.                                         */

/*                                                                       */

/*      6.     : If you change the path "C:\QB45\PC1ACOM.BAS" for the    */

/*               include file, you have to change the code related with  */

/*               the include file, and recompile and link.               */

/*-----------------------------------------------------------------------*/

/*                                                                       */

/*                              PC1ADEV.C                                */

/*                                          .                            */

/* Include file: PC1ACOM.BAS                                             */

/*                                                                       */

/* library file: PC1ALIB.QLB if run from inside the environment          */

/*               PC1ALIB.LIB if run outside the environment              */

/*                                                                       */

/* This demo program makes the PC1A card function as a device that sends */

/* out and receives data from the bus controller.                        */

/*                                                                       */

/* Data transfer is done by polling the 7210's address status register to*/

/* see whether my listen address or my talk address has been sent to the */

/* bus by the bus controller. If so, then the PC1A does a data transfer. */

/*                                                                       */

/* When you program  the bus controller, remember that an UNT has to be  */

/* sent after making the PC1A a talker and receiving its  data.          */

/*                                                                       */

/* Inside the program, the user can press the RETURN key to prompt for   */

/* keyboard input to the ring buffer. If the RETURN key is pressed and   */

/* the PC1A is not addressed as a talker,an SRQ will be generated to     */

/* interrupt the bus. Therefore the user can at his option program the   */

/* controller to handle a SRQ interrupt.                                 */

/*                                                                       */

/* ***REMARKS** :                                                        */

/*           1.  The program does not do  complete testing of all        */

/*               possible conditions that may happen . Because of this,  */

/*               you need to consider all exceptional cases in your      */

/*               program. In the interrupt service routine, do not wait  */

/*               for a condition which will not always or often occur.   */

/*               You need to pay extra attention to this, or else your   */

/*               program and  the whole system will be hung up by the    */

/*               interrupt service routine.                              */

/*                                                                       */

/*          2.   The PC1A can not be switched from a talker to a listener*/

/*               directly, the controller has to send a UNTALK before it */

/*               makes the PC1A a listener.                              */

/*                                                                       */

/*          3.   If you run from inside the Quick Basic development      */

/*               environment, you need to type QB /L PC1ALIB.QLB, then   */

/*               load QB1ADEV.BAS, and press ALT R and select Start.     */

/*                                                                       */

/*               If run from the DOS command line, just type QB1ADEV.    */

/*                                                                       */

/*          4.   The PC1A address switch setting is assumed to be set to */

/*               address 0xD000.                                         */

/*                                                                       */

/*          5.   If you change the path "C:\QB45\PC1ACOM.BAS" for the    */

/*               include file, you have to change the code related w/the */

/*               include file, and recompile and link.                   */

/*-----------------------------------------------------------------------*/

V.   THREE MORE DEMO PROGRAMS

(*-----------------------------------------------------------------------*)

(*                                                                       *)

(*        RE: DEMO PROGRAMS QB3478.BAS, QB33DEMO.BAS,and QB34DEMO.BAS    *)

(*        -----------------------------------------------------------    *)

(*                                                                       *)

(*     This document shows you how to run the QBasic demo programs       *)

(*     under Microsoft quick basic complier versions 3.0. and 4.0        *)

(*     or above.                                                         *)

(*                                                                       *)

(*     The demo programs are for the 4833, 4834 and HP3748A DMM          *)

(*     using an ICS 488-PC1A card.                                       *)

(*                                                                       *)

(*     In the programs, there are instructions in the form               *)

(*                        color n,n                                      *)

(*     used for setting the screen color.  If you use a monochrome       *)

(*     monitor, you have to delete them and recompile the program        *)

(*     to make it work.                                                  *)

(*                                                                       *)

(*************************************************************************)

0. This program sets the DEF SEG =&HD000 for the ICS's 488-PC1A card.

If you want to use a segment address other than D000, you have to

change the segment address in the program and change the segment

switch on your 488-PC1A card.

1. Set the address switches of the 4833 OR 4834 to 4 and the HP3478A to 2.

2. Copy the demo programs and (if using QBASIC 3.0) the file 'uselib1.exe'

into the directory with your Quick basic complier.

3. For Version 3.0 : Type the command " QB (your filename)/l uselib1.exe "

after the promp to enter into the 3.0 quick basic environment and compile

your program with the user library supplied by ICS, then run the program.

4. For Version 4.0 or above : Type the command " QB /L " after the prompt to

enter into the quick basic environment, then load the program and run it.

5. These programs have been developed to demonstrate certain features of

the ICS products. They may be used or changed at the users discression.

ICS DOES NOT WARRENT THAT THESE PROGRAMS WILL PERFORM UNDER ALL VERSIONS

OF QUICK BASIC OR IN ALL "IBM COMPATIBLE" COMPUTERS.

Copyright ICS Electronics Corporation 1989