README.DOC

**********

DISK P/N 123007  488-PC1A C LIBRARY DISK       Version 1   Dated 10/25/89

INTRODUCTION:

This disk contains C  language drivers and demonstration programs for

ICS's Model PC1A IEEE 488 Interface Card.  The C language drivers

provide libraries for the following C languages:

DISK CONTENTS:

***********************  LIBRARY NAME  **************************

C86 :

C86GPIBS.LIB    IEEE-488 library for small model

C86GPIBM.LIB    IEEE-488 library for medium model

C86GPIBB.LIB    IEEE-488 library for big model

Lattice C :

LCGPIBS.LIB     IEEE-488 library for S model

LCGPIBP.LIB     IEEE-488 library for P model

LCGPIBD.LIB     IEEE-488 library for D model

LCGPIBL.LIB     IEEE-488 library for L model

Microsoft C:

SMSCGPIB.LIB    IEEE-488 library for S model

MMSCGPIB.LIB    IEEE-488 library for M model

CMSCGPIB.LIB    IEEE-488 library for C model

LMSCGPIB.LIB    IEEE-488 library for L model

Turbo C :

STBCGPIB.LIB    IEEE-488 library for S model

MTBCGPIB.LIB    IEEE-488 library for M model

CTBCGPIB.LIB    IEEE-488 library for C model

LTBCGPIB.LIB    IEEE-488 library for L model

HTBCGPIB.LIB    IEEE-488 library for H model

***********************  DEMO PROGRAMS **************************

PROGRAM       DESCRIPTION

-------       -----------

*****require MSC 5.0/TURBO C 1.0:

PC1A_34.C      Demo data transfer, serial polling, and parallel polling

PC1AINT.C      Inputting data with SRQ interrupts

PC1ADEV.C      Operation as a device for data transfer

C86:

EXAMP1.C      Interactive data transfer

EXAMP2.C      Set the IEEE-488 printer

EXAMP3.C      Voltage measurement with DVM

EXAMP4.C      AD500 PMU programming

EXAMP5.C      Multiple device triggering

EXAMP6.C      Interrupt handling

The minimum host system to run Microsoft C 5.1  requires 512 kbytes

of RAM, DOS 2.1 or higher, and either twin 5 1/4-inch floppy drives or

a double-sided 3 1/2 inch microfloppy. Availability of a hard disk and

DOS 3.3 is highly recommended.

The minimum host sytem to run Microsoft C 5.0 is about the same as that

for 5.1, except that only 488 kbytes of RAM is required.

The minimum requirements for Turbo C 2.0 requires the IBM PS/2 or the

IBM family of personal computers and all 100% compatibles, PC-DOS

(MS-DOS) 2.0 or later, and at least 448 Kbytes of RAM.

The minimum requirements for Turbo C 1.0 requires the same as 2.0.

demo programs: pc1a_34.c, pc1aint.c pc1adev.c

----------------------------------------------

Above demo programs show how a PC/XT/AT with the PC1A card can be used as

a bus controller and as a bus device.  The fist demo program outputs

and reads back a data string.  The second program shows how the PC can

be programmed to sense SRQ interrupts and input data in response to the

SRQ line.  The third program shows how the PC can be programmed as a

device to respond to another IEEE 488 bus controller.

CAUTIONS:

For any of these three demo programs, pc1a_34.c, pc1aint.c, and

pc1adev.c, to work properly, you need to pay attention to the

followings:

1.  All three programs use ANSI device driver for the screen i/o.

A command in the form of device=pathname\ansi.sys needs to be

put into the config.sys file which is used during boot up time.

2.  If you use the demo program source code to develope your own

programs, note that these four string literals, TALK_S, LISTEN_S,

SEC0DATA_S, SEC0_S, always end up with exactly one space.

3.  PC1A memory segment switch  has to be set to hex d000.

/*-----------------------------------------------------------------------*/

/*                                                                       */

/*                              PC1A_34.C                                */

/*                                                                       */

/*               DATA LOOP BACK TEST OF A 4834 SERIAL INTERFACE UNIT     */

/*               CONTROLLED BY A PC1A CARD                               */

/*                                                                       */

/*  Purpose    : This program sends out data string to a channel of 4834,*/

/*               and reads the looped back data from the same channel and*/

/*               display it.                                             */

/*                                                                       */

/*               This program is written for the ICS Model 4834 Serial   */

/*               Interface which is used to connect devices with RS-232  */

/*               or RS-449 serial interface to the IEEE 488 bus. The T1  */

/*               command puts the 4834 into a local loop back test mode. */

/*                                                                       */

/*                                                                       */

/*  Remark     : Please pay attention to the following, or else it will  */

/*               not work properly.                                      */

/*                                                                       */

/*      1.     : In the program, "T1" is sent to the secondary address of*/

/*               4834 to select the local loop mode, which is necessary  */

/*               for this test.  Don't change it to "T0".                */

/*                                                                       */

/*-----------------------------------------------------------------------*/

/*                                                                       */

/*                              PC1AINT.C                                */

/*                                                                       */

/* This program demonstrates how a 4834 uses SRQ to interrupt PC/AT. It  */

/* uses interrupt vector 0x0f for irq7. The vector is redirected to point*/

/* to the new interrupt service routine.                                 */

/*                                                                       */

/* If you run this program under Microsoft C, keep the statement         */

/* #define  MSC  . If you run under Turbo C,  then you have either       */

/* delete these statements, or mark these statements as comments.        */

/* Then compile and link with other programs to produce the .exe file.   */

/*                                                                       */

/*      1.     : This program uses ANSI device driver for the screen.    */

/*               A command in the form of device=pathname\ansi.sys needs */

/*               to be put into the config.sys file which is used during */

/*               boot up time.                                           */

/*                                                                       */

/*      2.     : If you copy the demo source code,note these four strings*/

/*               TALK_S, LISTEN_S,SEC0DATA_S, and SEC0_S, these four     */

/*               strings always end up with exactly one space.           */

/*                                                                       */

/*      3.     : PC1A card switch setting has to be set to address 0xd000*/

/*                                                                       */

/*-----------------------------------------------------------------------*/

/*                                                                       */

/*                              PC1ADEV.C                                */

/*                                          .                            */

/*                                                                       */

/* This demo program makes PC1A card functin as a device that sends out  */

/* data and receives data from the bus controller.                       */

/*                                                                       */

/* Data transfer is done by polling the 7210 address status register to  */

/* see whether my listen address or my talk address has been sent to the */

/* bus by the bus controller. If so, then the PC1A does a data transfer. */

/*                                                                       */

/* When you program for the bus controller, remember that a UNT has to be*/

/* sent after making PC1A a talker and receiving its talked data.        */

/*                                                                       */

/* Inside the program, user can hit RETURN key to request the   keyboard */

/* input to the output ring buffer.  If RETURN key is hit and PC1A is not*/

/* addressed as a talker,an SRQ will be generated to interrupt controller*/

/* So a service routine needs to be programmed for the controller side.  */

/*                                                                       */

/* ***REMARK** :                                                         */

/*           1.  The program does not do a complete testing of all       */

/*               possible conditions that may happen .                   */

/*               But, you need to consider all exceptional cases in your */

/*               program. In the interrupt service routine, do not wait  */

/*               for a condition which will not always be the case.      */

/*               You need to pay extra attention to this, or else your   */

/*               program and even the whole system will hang up by the   */

/*               interrupt service routine.                              */

/*                                                                       */

/*          2.   pc1a can be switched from listener to talker directly   */

/*                                                                       */

/*          3.   pc1a can not be switched from a talker to listerner, the*/

/*               controller has to send a UNTALK before make pc1a a      */

/*               listener.                                               */

/*                                                                       */

/*-----------------------------------------------------------------------*/

Copyright ICS Electronics Corporation 1989