Interfacing TCL and the ICS 80xx Devices

The TCL demo package consists of four files.

- Makefile.dat

- Vxi.c

- Vxi11.x

- Demo.tcl

The makefile creates a vxi.so file which is combined with the demo.tcl to provide VXI-11 (RPC) functionality. The actual VXI-11 function definitions are found in the vxi.c file which provides the interface to the RPC layer.

Examination of the demo.tcl file first shows a connection to the 8065 gateway and the generation of an IFC signal. It then establishes a connection to the device at GPIB address 8, after which it sends an *IDN query and reads back the reply.

The Vxi_[function] calls are resolved in the vxi.c I/O library. Each of the functions must be defined at this layer, so as to establish RPC communication with the 8065. Each function definition in the I/O library consists of three basic elements.

- First the RPC VXI-11 command must be constructed. This consists of filling in RPC structure elements as required for the VXI-11 function to be performed.

- The RPC VXI-11 command is then sent to the target RPC service (8065).

- The RPC VXI-11 reply is then read from the target RPC service (8065).

The vxi.c file provided is a minimal demonstration program with only a few VXI-11 functions implemented. However it can be seen that adding additional VXI-11 functions would be relatively easy to accomplish. The existing vxi.c demo currently contains the following VXI-11 functions.

  - Vxi_Init (create the TCL wrapper for the VXI-11 support library)

  - Vxi_Open (establish an RPC connection and create a VXI-11 link)

  - Vxi_Close (destroy a device link and close the RPC connection)

  - Vxi_Ifc (issue an IFC pulse)

  - Vxi_Clear (perform a device clear)

  - Vxi_Read (read from a GPIB device)

  - Vxi_Write (write to a GPIB device)

It should be noted that the sample vxi.c file is a minimalistic example with little provided in the way of error checking. A true I/O support library would need to provide additional error detection and error handling. This is true for both the incoming parameters as well as the RPC and VXI-11 communication.